# Gas Market Report

## 13 – 19 December 2015

# Weekly summary

Average prices were significantly higher than the previous week across all markets. The smallest price increase was in Sydney, at 31 per cent. Prices in Adelaide increased the most, at 67 per cent. Prices may have been influenced by an overall increase in gas flows towards Queensland’s LNG export facilities, and maintenance at Longford affecting gas supplies into Victoria.

Bellevue gas plant restarted production from 11 December. Around the same time, QCLNG shipments on the Wallumbilla to Gladstone pipeline to Curtis Island increased. While flows on the APLNG pipeline gradually decreased over the week, and GLNG flows decreased from around mid-week, these changes were small in comparison.

Planned maintenance at Longford significantly reduced the amount of gas it could supply into Victoria (see figure 1.5). The outage contributed to price increases towards the end of the week in both Victoria and Adelaide (see figures 1.1 and 3.1). On 18 December, the gas market prices peaked at $10/GJ in Adelaide and $7.73/GJ in Victoria. This is explained in further detail under ‘detailed market analysis’ below.

# Long term statistics and explanatory material

The AER has published an explanatory note to assist with interpreting the data presented in its weekly gas market reports. The AER also publish a range of longer term statistics on the performance of the gas sector including gas prices, production, pipeline flows and consumer demand.

# Market overview

Figure 1 sets out the average daily prices ($/GJ) in the Victorian Declared Wholesale Market (VGM or Victorian gas market) and for the Sydney (SYD), Adelaide (ADL) and Brisbane (BRI) Short Term Trading Market hubs (STTM) for the current week compared to historical averages.

## Average daily prices – all markets ($/GJ)

| Region | Victoria | Sydney | Adelaide | Brisbane |
| --- | --- | --- | --- | --- |
| 13 Dec - 19 Dec 2015 | 5.19 | 4.91 | 6.64 | 4.12 |
| % change from previous week | 41 | 31 | 67 | 63 |
| 15-16 financial YTD | 4.42 | 4.61 | 5.03 | 3.67 |
| % change from previous financial YTD | 27 | 48 | 43 | 109 |

Figure 2 compares average weekly gas prices, ancillary market payments and scheduled injections against historical averages for the Victorian gas market.

## Victorian Gas Market

|  | Price ($/GJ) | Ancillary payments ($000)* | BOD forecast demand quantity (TJ) |
| --- | --- | --- | --- |
| 13 Dec - 19 Dec 2015 | 5.19 | - | 371 |
| % change from previous week | 41 | - | -1 |
| 15-16 financial YTD | 4.42 | - | 654 |
| % change from previous financial YTD | 27 | - | 7 |

* Note: only positive ancillary payments, reflecting system constraints will be shown here.

More detailed analysis on the VGM is provided in section 1.

Figures 3 to 5 show average ex ante and ex post gas prices, Market Operator Service (MOS) balancing gas service payments together with the related daily demand quantities against historical averages for the Sydney, Adelaide and Brisbane STTM hubs, respectively.

## Sydney STTM

|  | Ex ante price ($/GJ) | Ex post price ($/GJ) | MOS payments ($000) | Ex ante quantity (TJ) | Ex post quantity (TJ) |
| --- | --- | --- | --- | --- | --- |
| 13 Dec - 19 Dec 2015 | 4.91 | 4.63 | 25.68 | 212 | 208 |
| % change from previous week | 31 | 64 | -55 | 5 | 12 |
| 15-16 financial YTD | 4.61 | 4.32 | 24.39 | 244 | 239 |
| % change from previous financial YTD | 48 | 36 | 70 | -7 | -10 |

## Adelaide STTM

|  | Ex ante price ($/GJ) | Ex post price ($/GJ) | MOS payments ($000) | Ex ante quantity (TJ) | Ex post quantity (TJ) |
| --- | --- | --- | --- | --- | --- |
| 13 Dec - 19 Dec 2015 | 6.64 | 7.72 | 5.28 | 46 | 53 |
| % change from previous week | 67 | 69 | 11 | -4 | -8 |
| 15-16 financial YTD | 5.03 | 5.18 | 9.37 | 68 | 70 |
| % change from previous financial YTD | 43 | 49 | -21 | 0 | 2 |

## Brisbane STTM

|  | Ex ante price ($/GJ) | Ex post price ($/GJ) | MOS payments ($000) | Ex ante quantity (TJ) | Ex post quantity (TJ) |
| --- | --- | --- | --- | --- | --- |
| 13 Dec - 19 Dec 2015 | 4.12 | 4.18 | 1.67 | 85 | 86 |
| % change from previous week | 63 | 71 | -14 | 0 | 6 |
| 15-16 financial YTD | 3.67 | 3.62 | 1.40 | 90 | 90 |
| % change from previous financial YTD | 109 | 135 | 17 | -42 | -42 |

More detailed analysis of the STTM hubs is found in sections 2 to 4.

Section 5 provides analysis on production and pipeline flows on the National Gas Bulletin Board (Bulletin Board), as well as gas powered generation (GPG) volumes in each state, and section 6 provides information on the Gas Supply Hub (GSH) at Wallumbilla.

# Detailed market analysis

MOS in Sydney

The MOS requirements in the Sydney STTM hub on 15, 16 and 19 December were largely the result of forecasting errors (see figure 2.4).

Longford maintenance and high prices in Adelaide and Victoria

From 17 December, planned maintenance at Longford significantly reduced the amount of gas it could supply into Victoria. The outage contributed to price increases towards the end of the week in both Victoria and Adelaide. On 18 December, the gas market prices peaked at $10/GJ in Adelaide and $7.73/GJ in Victoria. Most of the shortfall was sourced from the Iona underground gas storage facility.

In Adelaide, temperatures exceeded 40 degrees from 16 December. The high temperatures increased electricity demand, which in turn increased the demand for gas from gas powered electricity generators. This led to an increase in gas prices in Adelaide and a higher proportion of supply sourced from the Moomba to Adelaide Pipeline (MAP) (see figure 3.3).

Unscheduled backhaul deliveries occurred during the second half of the week in Adelaide on both SEAGas and the MAP. The backhaul may have been nominated to supply gas to generators located outside of the hub. Participants increased supply to the hub from the MAP to offset the unscheduled backhaul nominations from SEAGas. To a lesser extent unscheduled backhaul from the MAP was offset by participants by nominating additional supply to the hub from SEAGas.

The unscheduled backhaul in Adelaide resulted in ex post prices increasing on these days (relative to the ex ante prices) despite demand within the hub being over forecast (the demand errors can be observed as decrease MOS in figure 3.4).

In Victoria, higher temperatures also influenced an increase in GPG output supplying higher electricity demand in the region. This had the effect of reversing flows on the South West Pipeline, which switched from supplying Adelaide and began flowing towards Melbourne from 17 December. Flows on the pipeline reached 169.5 TJ on 18 December.

# Detailed Market Figures

## 13 – 19 December 2015

### Victorian Declared Wholesale Market

In the Victorian gas market, gas is priced five times daily at 6 am, 10 am, 2 pm, 6 pm and 10 pm. The imbalance weighted price on a gas day tends towards the 6 am price which is the schedule at which most gas is traded.

The main drivers of price are demand forecasts and bids to inject or withdraw gas from the market. Figures 1.1 to 1.4 below show the daily prices, demand forecasts, and injection/withdrawal bids for each of the five pricing schedules. Figure 1.5 provides information on which system injection points were used to deliver gas, in turn indicating the location and relative quantity of gas injection bids cleared through the market.

#### Prices by schedule

#### Demand forecasts

#### Injection bids by price bands

#### Withdrawal bids by price bands

#### Metered Injections by System Injection Point

Note that in figure 1.5, the last 8-hour schedule from 10 pm has been separated into two 4-hour blocks to provide a consistent comparison with earlier scheduled injection volumes.

### Sydney STTM

In each STTM hub, a daily gas price is calculated before the gas day (the ex ante price) and after the gas day (the ex post price). The main drivers of these prices are participant demand forecasts, and offers to inject or bids to withdraw gas traded at the hub. Divergences in ex ante and ex post prices for a gas day may occur due to differences in scheduled (forecast) and allocated (actual) quantities. Pipeline acronyms are defined in the user guide.

Market Operator Service balancing gas (MOS) payments arise because the amount of gas nominated on pipelines for delivery on a gas day will either exceed or fall short, by some amount, of the amount of gas consumed in the hub. In such circumstances, MOS payments are made to participants for providing a service to park gas on a pipeline or to loan gas from a pipeline to the hub.

Figures 2.1 and 2.2 show daily prices, demand, offers and bids. Figures 2.3 and 2.4 show gas scheduled and allocated on pipelines to supply the hub, indicating the location and relative quantity of gas offers across pipelines and also the amount of MOS allocated for each pipeline.

#### SYD STTM daily ex ante and ex post prices and quantities

|  | Sun | Mon | Tue | Wed | Thu | Fri | Sat |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ex ante price ($/GJ) | 3.80 | 4.96 | 5.10 | 4.90 | 5.21 | 5.40 | 5.00 |
| Ex ante quantity (TJ) | 183 | 212 | 217 | 232 | 239 | 203 | 194 |
| Ex post price ($/GJ) | 2.50 | 4.94 | 5.20 | 4.93 | 5.21 | 5.40 | 4.25 |
| Ex post quantity (TJ) | 173 | 205 | 222 | 240 | 235 | 201 | 179 |

#### SYD daily hub offers and daily hub bids in price bands ($/GJ)

#### SYD net scheduled and allocated gas hub supply (excluding MOS)

#### SYD MOS allocations (TJ), service payments and commodity payments/charges ($000)

### Adelaide STTM

The Adelaide STTM hub functions in the same way as the Sydney STTM hub.  The same data that was presented for the Sydney hub is presented for the Adelaide hub in the figures below.

#### ADL STTM daily ex ante and ex post prices and quantities

|  | Sun | Mon | Tue | Wed | Thu | Fri | Sat |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ex ante price ($/GJ) | 4.25 | 4.80 | 5.30 | 5.40 | 8.33 | 10.00 | 8.40 |
| Ex ante quantity (TJ) | 40 | 48 | 49 | 47 | 51 | 49 | 39 |
| Ex post price ($/GJ) | 4.25 | 4.90 | 5.36 | 6.32 | 10.04 | 13.20 | 9.95 |
| Ex post quantity (TJ) | 40 | 52 | 52 | 58 | 68 | 55 | 46 |

#### ADL daily hub offers and daily hub bids in price bands ($/GJ)

#### ADL net scheduled and allocated gas hub supply (excluding MOS)

#### ADL MOS allocations (TJ), service payments and commodity payments/charges ($000)

### Brisbane STTM

The Brisbane STTM hub functions in the same way as the Sydney STTM hub.  The same data that was presented for the Sydney hub is presented for the Brisbane hub in the figures below.

#### BRI STTM daily ex ante and ex post prices and quantities

|  | Sun | Mon | Tue | Wed | Thu | Fri | Sat |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Ex ante price ($/GJ) | 3.51 | 5.50 | 4.07 | 4.16 | 4.48 | 4.00 | 3.11 |
| Ex ante quantity (TJ) | 74 | 92 | 91 | 90 | 88 | 85 | 73 |
| Ex post price ($/GJ) | 3.85 | 5.50 | 3.60 | 4.16 | 5.47 | 3.60 | 3.11 |
| Ex post quantity (TJ) | 79 | 90 | 90 | 91 | 91 | 84 | 76 |

#### BRI daily hub offers and daily hub bids in price bands ($/GJ)

#### BRI net scheduled and allocated gas hub supply (excluding MOS)

#### BRI MOS allocations (TJ), service payments and commodity payments/charges ($000)

### National Gas Bulletin Board

Figure 5.1 shows average daily actual flows for the current week from the Bulletin Board (changes from the previous week’s average are shown in brackets). Average daily scheduled volumes and prices are provided for gas markets and gas powered generation for each region.

#### Gas market data ($/GJ, TJ/day); Production, Consumption and Pipeline flows (TJ)

### Gas Supply Hub

The Gas Supply Hub (GSH) was established for the trading of gas at Wallumbilla because it is located in close proximity to significant gas supply sources and demand locations and is a major transit point between Queensland and the gas markets on Australia’s east coast. The GSH is a voluntary market for the supply of gas traded between separate participants, with products listed for sale and purchase at delivery points on three major connecting pipelines at Wallumbilla – the Queensland Gas Pipeline (QGP), the South West Queensland Pipeline (SWQP) and the Roma to Brisbane Pipeline (RBP). There are separate products for each pipeline (each pipeline is considered a trading location, and each has a number of delivery points) and delivery period (daily, day-ahead, balance-of-day and weekly).

There were 35 trades for 374 TJ of gas this week at a volume weighted price of $3.83/GJ. While there was one weekly trade on the RBP this week, the majority of the gas traded was through daily, day-ahead and balance-of-day products on the SWQP at a volume weighted price of $3.79/GJ (281 TJ traded). The volume weighted price for the RBP was $3.97/GJ (93 TJ traded).

Figure 6.1 shows volumes traded on each gas day and trading day for the current week.

#### Volume Traded (by Gas Day and by Trading Day)

Bellevue is a gas plant that supplies Queensland Curtis’ LNG’s export project in Queensland.

Average daily production from Bellevue was around 186 TJ over the week, while daily export flows on the Wallumbilla to Gladstone Pipeline (used by QCLNG) roughly doubled to around 1500 TJ – over four times the size of Victoria’s average beginning of day demand forecast for the week of 371 TJ.

The Moomba to Sydney Pipeline also saw its first negative flows reported for the Moomba zone around the same time. Flows on the QSN Link reversed direction, and transported gas towards Queensland during the week.

The weighted average daily imbalance price applies for Victoria.

Prices in Adelaide increased significantly beyond the provisional schedules’ forecasts. This was influenced by the rebidding of supply offers into higher prices on both the Moomba to Adelaide pipeline and the SEAGas pipeline. The most significant impact came from the higher priced SEAGas supply, which was influenced by the higher price of gas coming out of Victoria.

Outlet pressure at the Dandenong City Gate was lowered to 2600 kPa to increase supply capacity on the South West Pipeline. This allows for additional supply into Melbourne from production facilities in the Otway region (including output from Iona). Longford’s maximum output over the period reduced from around 440 TJ/d to being constrained as low as 109 TJ/d on 19 December.

The estimated requirement for gas powered generation in South Australia increased above 257 TJ on the 16 December gas day and remained high for the remainder of the week (this is evident in the average daily increase in GPG shown in figure 5.1).

Backhaul provides gas to users located outside of the hub. SEAGas and the MAP provided daily amounts of up to 20 TJ and 10 TJ of unscheduled backhaul respectively in the second half of the week.

Temperatures in Melbourne increased to 37 degrees on 17 December and climbed to a maximum of 41 degrees on 19 December. Over the same period, peak electricity demand in the state increased from 6198 MW on 14 December to a maximum of 8566 MW on 18 December.

Prices for subsequent schedules are applied only to the differences in scheduled quantities (imbalances) to calculate the weighted price. The 6 am price applies to the entire scheduled quantity in the initial schedule.

The price might also be affected by transmission or production (contractual) constraints limiting how much gas can be delivered from a locale or System Injection Point (SIP) from time to time.

These are Market Participants’ aggregate demand forecasts adjusted for any override as applied by AEMO from time to time. These forecasts must be scheduled and cannot respond to price like withdrawal bids.

The main driver of the amount of gas scheduled on a gas day is the ‘price-taker’ bid, which is forecast hub demand that cannot respond to price and which must be delivered, regardless of the price.

MOS service payments involve a payment for a MOS increase service when the actual quantity delivered exceeds final gas nominations for delivery to a hub, and a payment for a MOS decrease service when the actual quantity delivered is less than final nominations. As well as a MOS ‘service’ payment, as shown in figure 2.4, MOS providers are paid for or pay for the quantity of MOS sold into the market or bought from the market (MOS ‘commodity’ payments/charges).

Gas flows shown under regional headings:  SA = MAP + SEAGAS, VIC = SWP + LMP – negative(NSW-VIC),

NSW/ACT = EGP + MSP, TAS = TGP, QLD (Brisbane) = RBP, QLD (Mt Isa) = CGP, QLD (Gladstone) = QGP

GPG volumes may include gas usage that does not show up on Bulletin Board pipeline flows.

Roma included export LNG production from October 2014 and LNG pipeline flows are shown from October 2015.

Wallumbilla supply is the average daily volume of gas ‘traded’, while price is a volume weighted average.

Market trade is facilitated through an electronic trading platform, with standardised terms and conditions and a market settlement facility for the short-term trading of physical gas and related products. The market is designed to complement existing bilateral gas supply arrangements and gas transportation agreements, through the placement of anonymous offers (to sell) or bids (to buy) at specified quantity and price increments, which are automatically matched on the exchange to form transactions.

Volumes shown for weekly products include the ‘daily’ volume for each relevant ‘gas day’, and the ‘weekly’ volume for each relevant ‘trading day’.